1.Parameters of servo motor

- Stroke: 500mm

- Speed: 3000 laps/min

- Load: 700N

- Lap: 50; When the motor runs for 1 laps, the electric actuator will extend by 10mm

- Communication frequency: 250k ; Because the communication frequency of the joystick is 250k, and the joystick and the motor use the same bus, the frequencies of the two must be kept consistent.

- In order to change the communication frequency, prepare a RS485 to USB module in advance.

actuator

.

2.Description of lib

- CANServo.h:  It was written by Han Long, but I didn't use it.

- Gripper.h: I wrote the code from line 89 to the end. class Gripper is used for the old machine.

- HAXTongyi: It is an example provided by the manufacturer.

- Servo: function of controlling servo motor

- Joystick.cpp: controlCylinders() is used for testing

- 3.Process of controlling the motor

- Step1: Turn on the motor power and then turn on the EE box power.

- Note: The controller of the motor can only receive the NMT command from the PCB after it is started, and can only start CAN communication after successfully receiving the command.

- Step2: Wait

- Note: use the current code, after about twenty seconds, the motor will be enabled, and after another ten seconds, you can control the motor using the joystick.

- The red LED is an alarm light.  When it flashes, it indicates that the motor or controller is running in error.  The current solution is to power off and restart.

- The green LED is the controller operation indicator.  When it is always on, it indicates that the motor is already in the enabled state.

- Step3:  control

- Use PDO; position Mode

- Step4: After operating the actuator to 0 position(Stroke: 500mm), turn off the power

- Note: The motor currently used is a relative value motor.

- code

PIN EXTEND

PIN SECOND_CUTTER

PIN GRIPPER_POS & PIN GRIPPER_NEG

PIN ROTATION

  - definition of pin

  - Sequence

  - Time control of sequence

  - Set the limit position of the motor

  - Set the speed of the motor

Generally set the motor to run at the fastest speed.

  - Feedback value of motor